University of Jordan

Department of Computer Engineering

Digital Logic Laboratory

Lab Project

Due Date (30/4/2017)

The purpose of this project is to design a 4-bit adder/subtractor.  The adder/subtractor should perform signed and unsigned add and subtract operations. The block diagram of the adder/subtractor is shown in Figure 1.

Figure 1: Addrer/Subtractor Block Diagram

- Adder/Subtractor Inputs

The adder/subtractor has two data inputs A(A3A2A1A0) and B(B3B2B1B0) and the operation M(M1M0). Based on the value of M the values of A and B will be added or subtracted.  Table 1 shows the operation for each value of M.

| M0 | M1 | Operation |
| --- | --- | --- |
| 0 | 0 | Unsigned add |
| 0 | 1 | Unsigned subtract |
| 1 | 0 | Signed add |
| 1 | 1 | Signed subtract |

Table 1: Adder/Subtractor operations

The data inputs to the adder/subtractor module will be read from two 4 bits registers. Register A will have the value of A. Register B will have the value of B.  The values at the inputs of the registers will be controlled using the switches on the FPGA. When the Load Switch is high, the registers will be loaded with the values represented by the switches. The Load signal will used to trigger the Parallel load control signal of the registers to perform this task.

After the registers are loaded with A and B values, the adder/subtractor is supposed to perform the operation specified by M which is also controlled by two switched on the FPGA.

- Adder/Subtractor Outputs

After the adder/Subtractor module finishes execution then the result will be displayed on a 7 segment display. In addition to output of the performed operation the adder/subtractor will generate the overflow and zero outputs, each of them will be displayed on an LED. Table 2 shows the description of the overflow and the zero outputs.

| Overflow | =1 The performed operation has an overflow |
| --- | --- |
|  | =0 The performed operation does not have an overflow |
| Zero | =1 The result of the performed operation is 0 |
|  | =0 The result of the performed operation is not Zero |

Table 2: Details of the Overflow and Zero outputs

- Display of Results

To make it easier the inputs and the outputs values will be displayed on 7 segment displays. Figure 2 and Table 3 show the details of how the data is displayed on the 7 segment displays for each operation.

| Sign A | Display A | Sign B | Display B | Display M | Sign R | Display R |
| --- | --- | --- | --- | --- | --- | --- |

Figure 2: Names and order of the 7-segment displays

| Operands A and B |  |  |  |
| --- | --- | --- | --- |
| Sign | Signed Add/subtract | Displays "-" when the number is negative OFF when the number is positive |  |
|  | Unsigned Add/subtract | Always OFF |  |
| Display | Unsigned Add/subtract | Display the value of the operand (Between 0-F) |  |
|  | Signed Add/subtract | Display the absolute value of the operand. Example: If A = 1111 then A Display should show 1 = ¦-1¦ If B = 0111 then B Display should show 7 = ¦7¦ |  |
| Result (R) |  |  |  |
| Sign | unsigned Add/subtract | Display "-" if A < B (borrow bit =1) OFF if A>=B (borrow bit =0) |  |
|  | signed Add/subtract | Display "-" when the number is negative OFF when the number is positive |  |
| Display | unsigned Add/subtract | No borrow: Display the value of R (Between 0-F) Borrow: Display the 2's complement of R. |  |
|  | signed Add/subtract | Display the absolute value of R Example: If R3R2R1R0 = 1111 then R Display should show 1 = ¦-1¦ If R3R2R1R0 = 0111 then R Display should show 7 = ¦7¦ |  |
| Operation (M) |  |  |  |
| Display | All operations |  | Display the value of M 00 0 01 1 10 2 11 3 |

Table 3: Design specifications

- Submission Guidelines

- Students are allowed to work in teams of two and students from different sections can work in the same team.

- Each team should email all the project files to their lab engineer before the deadline.

  - Eng. Alaa Thaher           Al.thaher@ju.edu.jo

  - Eng. Rawan Al-jamal     R.aljamal@ju.edu.jo

  - Eng. Amal Quzmar        A.quzmar@ju.edu.jo

- Each team should be able to show a demo of the working project to their instructor and lab engineer during their lab section.

- Students are expected to answer all the questions during the discussion.

- Cheating will not be tolerated.

- Pin Assignments

1- R Display (HEX1: Sign, HEX0: Display)

| a1 | oHEX1_D[0] | PIN_AG13 |
| --- | --- | --- |
| b1 | oHEX1_D[1] | PIN_AE16 |
| c1 | oHEX1_D[2] | PIN_AF16 |
| d1 | oHEX1_D[3] | PIN_AG16 |
| e1 | oHEX1_D[4] | PIN_AE17 |
| f1 | oHEX1_D[5] | PIN_AF17 |
| g1 | oHEX1_D[6] | PIN_AD17 |

| a0 | oHEX0_D[0] | PIN_AE8 |
| --- | --- | --- |
| b0 | oHEX0_D[1] | PIN_AF9 |
| c0 | oHEX0_D[2] | PIN_AH9 |
| d0 | oHEX0_D[3] | PIN_AD10 |
| e0 | oHEX0_D[4] | PIN_AF10 |
| f0 | oHEX0_D[5] | PIN_AD11 |
| g0 | oHEX0_D[6] | PIN_AD12 |

2- B Display (HEX5: Sign, HEX4: Display)

| a5 | oHEX5_D[0] | PIN_M3 |
| --- | --- | --- |
| b5 | oHEX5_D[1] | PIN_L1 |
| c5 | oHEX5_D[2] | PIN_L2 |
| d5 | oHEX5_D[3] | PIN_L3 |
| e5 | oHEX5_D[4] | PIN_K1 |
| f5 | oHEX5_D[5] | PIN_K4 |
| g5 | oHEX5_D[6] | PIN_K5 |

| a4 | oHEX4_D[0] | PIN_P1 |
| --- | --- | --- |
| b4 | oHEX4_D[1] | PIN_P2 |
| c4 | oHEX4_D[2] | PIN_P3 |
| d4 | oHEX4_D[3] | PIN_N2 |
| e4 | oHEX4_D[4] | PIN_N3 |
| f4 | oHEX4_D[5] | PIN_M1 |
| g4 | oHEX4_D[6] | PIN_M2 |

3- A Display (HEX7: Sign, HEX6: Display)

| a7 | oHEX7_D[0] | PIN_K3 |
| --- | --- | --- |
| b7 | oHEX7_D[1] | PIN_J1 |
| c7 | oHEX7_D[2] | PIN_J2 |
| d7 | oHEX7_D[3] | PIN_H1 |
| e7 | oHEX7_D[4] | PIN_H2 |
| f7 | oHEX7_D[5] | PIN_H3 |
| g7 | oHEX7_D[6] | PIN_G1 |

| a6 | oHEX6_D[0] | PIN_H6 |
| --- | --- | --- |
| b6 | oHEX6_D[1] | PIN_H4 |
| c6 | oHEX6_D[2] | PIN_H7 |
| d6 | oHEX6_D[3] | PIN_H8 |
| e6 | oHEX6_D[4] | PIN_G4 |
| f6 | oHEX6_D[5] | PIN_F4 |
| g6 | oHEX6_D[6] | PIN_E4 |

4- M Display (HEX3)

| a3 | oHEX3_D[0] | PIN_P6 |
| --- | --- | --- |
| b3 | oHEX3_D[1] | PIN_P4 |
| c3 | oHEX3_D[2] | PIN_N10 |
| d3 | oHEX3_D[3] | PIN_N7 |
| e3 | oHEX3_D[4] | PIN_M8 |
| f3 | oHEX3_D[5] | PIN_M7 |
| g3 | oHEX3_D[6] | PIN_M6 |

5- Overflow and Zero pins:

| Overflow | oLEDG[7] | PIN_AA24 |
| --- | --- | --- |
| Zero | oLEDR[0] | PIN_AJ6 |

6- Input Switches (A and B Operands):

| A0 | iSW[1] | PIN_AB26 |
| --- | --- | --- |
| A1 | iSW[2] | PIN_AB25 |
| A2 | iSW[3] | PIN_AC27 |
| A3 | iSW[4] | PIN_AC26 |
| B0 | iSW[14] | PIN_L5 |
| B1 | iSW[15] | PIN_L4 |
| B2 | iSW[16] | PIN_L7 |
| B3 | iSW[17] | PIN_L8 |
| Load | iSW[0] | PIN_AA23 |
| M0 | iSW[10] | PIN_W5 |
| M1 | iSW[11] | PIN_V10 |
| Clock |  | PIN_E16 |