University of Jordan

Faculty of Engineering and Technology

Department of Computer Engineering

Digital Logic Laboratory 0907234

Labsheet 7:   Arithmetic Circuits Design and Implementation

Name:

Student ID:

Section:

You are required to design an arithmetic circuit which implements the following operations:

Addition (+), Subtraction (-), Multiplication (*) depending on OP0 and OP1 bits as shown in the table:

| Arithmetic Operation | OP0 | OP1 |
| --- | --- | --- |
| Addition (+) | 0 | 0 |
| Subtraction (-) | 1 | 0 |
| Multiplication (*) | 0 | 1 |

Part1: 1-bit Full Adder design.

- Find the equation of outputs: 					(Pre Lab)

S=

COUT=

- Build the ' FA ' module using Verilog code structurally inside FA.v file. This module will have 3-inputs (X, Y and Z) and 2-outputs (S and COUT).                      	 				       (Pre Lab)

Part2: 2-bit Full Adder/Subtractor (ripple) design:

- Build the ' ripple ' module using Verilog code structurally inside ripple.v file. This module will have 5-inputs (a0, a1, b0, b1, M) and 3-outputs (S0, S1, COUT).    					(Pre Lab)

Part3: 2x2 Multiplier design.

- Refer to Experiment 7 tutorial page number 4. Build the ' Mul ' module using Verilog code structurally inside Mul.v file which represent 2-bit Multiplier with 4-inputs (a0, a1, b0, b1) and 4-outputs (C0,C1,C2,C3). 											(Pre Lab)

Note: instead of creating new module for the half adder use the FA (full adder module) and set the Z input as 0.

Part4: Final circuit design.

- Build the ' circuit ' module using Verilog code structurally inside circuit.v file. This represents whole circuit with:											(In Lab)

- 6-inputs (a0, a1, b0, b1, OP0, OP1) “connected to switches”.

- 28-Outputs:

- 7: ‘a, b, c, d, e, f, g ‘for the first operand (a) 7-segment’.

- 7: ‘a2, b2, c2, d2, e2, f2, g2 ‘for the second operand (b) 7-segment’.

- 7: ‘a3, b3, c3, d3, e3, f3, g3 ‘for the operation 7-segment’.

- 7: ‘a4, b4, c4, d4, e4, f4, g4 ‘for the Result 7-segment’.

- Make the pins assignment for your inputs and outputs. 					               (In Lab)

Note: Use the following pins for your inputs and outputs:

| Hex (0): Operand A |  |  |
| --- | --- | --- |
| a | oHEX0_D[0] | PIN_AE8 |
| b | oHEX0_D[1] | PIN_AF9 |
| c | oHEX0_D[2] | PIN_AH9 |
| d | oHEX0_D[3] | PIN_AD10 |
| e | oHEX0_D[4] | PIN_AF10 |
| f | oHEX0_D[5] | PIN_AD11 |
| g | oHEX0_D[6] | PIN_ AD12 |
| Hex (1): Operand B |  |  |
| a2 | oHEX1_D[0] | PIN_AG13 |
| b2 | oHEX1_D[1] | PIN_AE16 |
| c2 | oHEX1_D[2] | PIN_AF16 |
| d2 | oHEX1_D[3] | PIN_AG16 |
| e2 | oHEX1_D[4] | PIN_AE17 |
| f2 | oHEX1_D[5] | PIN_AF17 |
| g2 | oHEX1_D[6] | PIN_AD17 |
| Hex (2): Operation |  |  |
| a3 | oHEX2_D[0] | PIN_AE7 |
| b3 | oHEX2_D[1] | PIN_AF7 |
| c3 | oHEX2_D[2] | PIN_AH5 |
| d3 | oHEX2_D[3] | PIN_AG4 |
| e3 | oHEX2_D[4] | PIN_AB18 |
| f3 | oHEX2_D[5] | PIN_AB19 |
| g3 | oHEX2_D[6] | PIN_AE19 |
| Hex (3): Result |  |  |
| a4 | oHEX3_D[0] | PIN_P6 |
| b4 | oHEX3_D[1] | PIN_P4 |
| c4 | oHEX3_D[2] | PIN_N10 |
| d4 | oHEX3_D[3] | PIN_N7 |
| e4 | oHEX3_D[4] | PIN_M8 |
| f4 | oHEX3_D[5] | PIN_M7 |
| g4 | oHEX3_D[6] | PIN_M6 |
| Input Switches |  |  |  |
| a0 | iSW[1] | PIN_AB26 |
| a1 | iSW[2] | PIN_AB25 |
| b0 | iSW[3] | PIN_AC27 |
| b1 | iSW[4] | PIN_AC26 |
| OP0 | iSW[5] | PIN_AC24 |
| OP1 | iSW[6] | PIN_AC23 |

- Download your Verilog program on FPGA and test your code.                                             	          (In Lab)