t2

University of Jordan

Faculty of Engineering and Technology

Department of Computer Engineering

Digital Logic Laboratory 0907234

Labsheet 8:   Shift Register Application & Counter Design

Name:

Student ID:

Section:

Part1: Counter design.

You are required to design a 2-bit counter that counts 0,3,1,2,0,3,1,2,0,…….

- Fill the truth table below

- Write the equations of the D-Flip-flop's outputs (counter output).

| Present State |  | Next State |  |
| --- | --- | --- | --- |
| Q1 | Q0 | Q1 | Q0 | D1 | D0 |
| 0 | 0 |  |  |  |  |
| 0 | 1 |  |  |  |  |
| 1 | 0 |  |  |  |  |
| 1 | 1 |  |  |  |  |

D0=

D1 =

- Create a new project 'twobitcounter' in QuartusII, add files 'twobitcounter.bdf' ,'lib.v', 'segdecoder.v', 'segdecoder.bsf' and the clock files (clock.bdf, clock.bsf, counter.v & pll.v) to your project.

- Build you design of the counter inside the file 'twobitcounter.bdf'.

Notes:

- To add the D Flip-Flop to your file, select the symbol of 'AND' gate on the tool bar (from where you get the IC's usually) then select primitives  storage D Flip-Flop (dff).

- To add the 'segdecoder’ and ‘clock’ symbols:

  - Select the symbol of 'AND' gate on the tool bar (from where you get the IC's usually).

  - Select project from the menu opened and expand it, you will find your symbol there, select it and insert to the schematic file.

  - If the symbol does not appear under project menu, type the symbol name in the text box below.

- Clock input:

Clock should be obtained from the system (FPGA) clock, but this clock has a very high frequency, you can't detect the changes of outputs with such a frequency, so you should have a frequency division circuit to minimize it to an acceptable and notable one.

You can use the 'clock.bsf' block you add previously to your project to do that task, this block has a one input "os_clock " (system clock '28 MHZ' frequency) and one output "out_clock" (1-10HZ).

Connect the os_clock to input pin “clk “and the out_clock to your dffs clock.

- Make the pins assignment for your inputs and outputs.

Note: Use the following pins for your inputs and outputs:

| Input |  |  |
| --- | --- | --- |
| clk |  | PIN_E16 |
| Output of the Counter |  |  |
| a | HEX0 | PIN_AE8 |
| b | HEX0 | PIN_AF9 |
| c | HEX0 | PIN_AH9 |
| d | HEX0 | PIN_AD10 |
| e | HEX0 | PIN_AF10 |
| f | HEX0 | PIN_AD11 |
| g | HEX0 | PIN_AD12 |

- Download your circuit on FPGA and test your design.

Part2: Shift Register Application.

You are required to design the following 2-bit serial adder that performs the operation:  C = A + B

- Consider that A and B are 2-bit operands.

- The result should be stored in Register ‘C’.

Mode

Reset

Clk /Push Button

Register A

Register B

Q         D

A

B

Cin

Register C

S

Cout

Input A

Input B

Full Adder

Figure (1)

- Design details:

Register A

A1                A0

Reset

1

0

Parallel

Serial

Mode

Clock

D          Q

Out

Figure (2)                                                                                   Figure (3)

1) Create a new project 'serial_adder' in QuartusII, add files 'onebitReg.bdf' & 'serial_adder.bdf' to your project.

2) Open the file 'onebitReg.bdf' and make the design in figure (3).

Note:

- To add the D Flip-Flop to your file select the symbol of 'AND' gate on the tool bar (from where you get the IC's usually).

- Select primitives  storage D Flip-Flop (dff),

- To add 2-1Mux select others  maxplus2 21mux.

3) Convert the one bit design you made to a block like in figure (2):

(File  Create/update  Create symbol files for current file).

4) Open the file 'serial_adder.bdf', use the 'onebitReg' symbol you create in the previous step & the Full Adder (FA symbol) to make the design of serial adder (Figure1).

- To add the 'onebitReg' and FA symbols:

a) Select the symbol of 'AND' gate on the tool bar (from where you get the IC's usually).

b) Select project from the menu opened and expand it, you will find your symbol there, select it and insert to the schematic file.

Note: If the symbol does not appear under project menu, type the symbol name in the text box below.

5) Make the required pin assignment as follow:

*Inputs:

-A0: SW [1]  PIN _AB26                                        -A1: SW [2]  PIN _AB25

-B0: SW [10]  PIN_W5                                          -B1: SW [11]   PIN_V10

- Mode: SW [0]  PIN_AA23                                 - CLK/PB: PB0  PIN_T29

- Reset: SW [17]  PIN_L8

*Outputs:

-AA0: LEDR [1]  PIN_AK5                                           -AA1: LEDR [2]  PIN_AJ5

-BB0: LEDR [10]  PIN_AC13                                      -BB1: LEDR [11]  PIN_AB13

-CC0: LEDG [1]  PIN_W25                                          -CC1: LEDG [2]  PIN_W23

6) Download your design to FPGA and test it.